Answers for report …

Requirement 01

Part A – Section 1. Network Decisions and Justification

Requirement 1: Secure direct Ethernet link between EXEC and HR

Technology Implemented

To satisfy the requirement for secure communication of sensitive material between EXEC and HR, an IPSec VPN (Internet Protocol Security Virtual Private Network) in tunnel mode was implemented over the direct Ethernet link.

IPSec was chosen as it provides end-to-end encryption, data integrity, and peer authentication, ensuring that all traffic exchanged between EXEC and HR remains confidential and unaltered during transmission.

Implementation Decision and Explanation

Both EXEC and HR routers were configured to establish an IPSec tunnel using:

- ISAKMP (Internet Security Association and Key Management Protocol) Policy (Phase 1): Configured with AES encryption, SHA hashing, pre-shared key authentication, and Diffie-Hellman Group 2 for secure key exchange. This ensures that initial key negotiations are securely established between the two routers.

- Pre-shared Key: A shared secret (“SECURE_KEY”) was configured on both routers to authenticate each other before establishing the secure tunnel, providing a simple and effective authentication mechanism within an internal network context.

- IPSec Transform Set (Phase 2): Configured with ESP-AES for data encryption and ESP-SHA-HMAC for integrity. This combination ensures that all data packets transmitted over the link are encrypted to maintain confidentiality and hashed to ensure integrity.

- Crypto Maps and ACLs: Crypto maps were created on both routers binding the ISAKMP and IPSec policies to the FastEthernet0/0 interfaces. An ACL matching all traffic between EXEC and HR was applied to identify the ‘interesting traffic’ that requires encryption.

Justification

IPSec was selected as it is a mature, industry-standard protocol suite that ensures:

- Confidentiality through encryption of data packets

- Integrity and authenticity using hashing and pre-shared key authentication

- Compatibility with existing Cisco router capabilities, thus avoiding additional infrastructure costs

Furthermore, implementing IPSec over the direct Ethernet link provides an additional security layer even within an internal environment, addressing the company’s concern for protecting sensitive material communicated between these two departments.

Part A – Section 2. Router Configurations

Requirement 1: Secure direct Ethernet link between EXEC and HR

EXEC Router – Key Configuration Commands

cisco

CopyEdit

! Configure EXEC interface with IP address

interface FastEthernet0/0

ip address 10.10.10.1 255.255.255.252

no shutdown

! Enables the interface and assigns IP for EXEC-HR link

! Configure ISAKMP policy for Phase 1 negotiation

crypto isakmp policy 10

encr aes

hash sha

authentication pre-share

group 2

lifetime 86400

! Defines encryption (AES), hashing (SHA), pre-shared authentication, DH group 2, and lifetime

! Set the pre-shared key for HR router authentication

crypto isakmp key SECURE_KEY address 10.10.10.2

! Defines shared key for authentication with HR peer

! Configure IPSec transform set for Phase 2

crypto ipsec transform-set EXEC-HR-SET esp-aes esp-sha-hmac

! Defines ESP encryption using AES and integrity using SHA-HMAC

! Create crypto map binding peer, transform set, and ACL

crypto map EXEC-HR-MAP 10 ipsec-isakmp

set peer 10.10.10.2

set transform-set EXEC-HR-SET

match address 101

! Binds HR peer, transform set, and traffic selection ACL

! Define ACL for interesting traffic to encrypt

access-list 101 permit ip any any

! Matches all IP traffic between EXEC and HR for encryption

! Apply crypto map to EXEC interface

interface FastEthernet0/0

crypto map EXEC-HR-MAP

! Activates IPSec on the interface towards HR

HR Router – Key Configuration Commands

c

CopyEdit

! Configure HR interface with IP address

interface FastEthernet0/0

ip address 10.10.10.2 255.255.255.252

no shutdown

! Enables the interface and assigns IP for HR-EXEC link

! Configure ISAKMP policy for Phase 1 negotiation

crypto isakmp policy 10

encr aes

hash sha

authentication pre-share

group 2

lifetime 86400

! Defines encryption (AES), hashing (SHA), pre-shared authentication, DH group 2, and lifetime

! Set the pre-shared key for EXEC router authentication

crypto isakmp key SECURE_KEY address 10.10.10.1

! Defines shared key for authentication with EXEC peer

! Configure IPSec transform set for Phase 2

crypto ipsec transform-set HR-EXEC-SET esp-aes esp-sha-hmac

! Defines ESP encryption using AES and integrity using SHA-HMAC

! Create crypto map binding peer, transform set, and ACL

crypto map HR-EXEC-MAP 10 ipsec-isakmp

set peer 10.10.10.1

set transform-set HR-EXEC-SET

match address 101

! Binds EXEC peer, transform set, and traffic selection ACL

! Define ACL for interesting traffic to encrypt

access-list 101 permit ip any any

! Matches all IP traffic between HR and EXEC for encryption

! Apply crypto map to HR interface

interface FastEthernet0/0

crypto map HR-EXEC-MAP

! Activates IPSec on the interface towards EXEC